PRODUCTS SPECIFICATION

FLLED30MP 30W LED Floodlight

(110V 30W minipod LED light)

Product name:          LED FloodLight

Item No     :              FLLED30MP

Power			:              30Watt

LED        :        1 pc COB x 30Watt

Input Voltage:           AC 110Voltage

【Datasheet 】

| Type | Product | LED FloodLight |  |  |
| --- | --- | --- | --- | --- |
|  | Modle No | FLLED30MP |  |  |
|  | Product Type | outdoor Lighting |  |  |
|  | Safety Level | Class I |  |  |
|  | IP Level | IP44 |  |  |
|  | Safety Certificates | CE |  |  |
|  | RoHS Directive | Compliant |  |  |
| Electronical | Input Voltage | 110 AC |  |  |
|  | Frequency | 50/60Hz |  |  |
|  | Power | 30Watt |  |  |
|  | Power factor( PF) | ＞90％ |  |  |
|  | Total Harmonic Distortion | ＜20％ |  |  |
| Optical | Cover Material | Tempered glass lens |  |  |
|  | The whole lamp light Angle | 120° |  |  |
|  | LED Quantity | 1 |  |  |
|  | LED Package | COB |  |  |
|  | Luminous Efficiency(Lm/W) | ≥80Lm/W |  |  |
|  | LED luminous | Warm white | 2559LM |  |
|  |  | Pure white | 2860LM |  |
|  |  | Cold white | 2790LM |  |
|  | Color temperature(CCT) | Warm white | 2700-3500K |  |
|  |  | Pure white | 5500-6500K |  |
|  |  | Cold white | 7000-10000K |  |
|  | Color Render Index(CRI) | W W | Ra＞78 |  |
|  |  | C P W | Ra＞85 |  |
|  | Illuminance uniformity | ＞0.6 |  |  |
|  | Light distribution | Symmetric/rectangular flare |  |  |
|  | Light design | LED + reflectors secondary light distribution |  |  |
| Lamp Structure | Housing Material | Die-casting Aluminum body, powder coating |  |  |
| Others | Application | Outdoor or indoor |  |  |
|  | LED Junction temperature | ≤60℃ |  |  |
|  | Ambient Humidity | 10％~90％R.H. |  |  |
|  | Ambient Temperature | -25℃- 55℃ |  |  |
|  | Storage Tempertature | -30℃- 75℃ |  |  |
|  | Housing Tempertature（Working） | ＜60℃（Ta=25℃） |  |  |
|  | Lifespan | 50000H |  |  |
|  | Color of lamp shell | Yellow, black |  |  |
| Dimension | Power line | 3G1.5mm2 3mtr cable |  |  |
|  | Connect wire | Brown |  | L |
|  |  | Blue |  | N |
|  |  | Yellow/Green |  | E |
| Carton Box Dimension | Quantity of each Carton | 5PCS |  |  |
|  | Net Weight | 13.8kg |  |  |
|  | Gross Weight | 16kg |  |  |
|  | Inner Box Dimension | 28X14X30.5cm |  |  |
|  | Outer Box Dimension | 72x30x33cm |  |  |

【Light Distribution】